# UAV Powertrain Acoustic Monitoring: Decision Framework

## Table 1: Questions and Alternatives

| Question | Decision | Rejected Alternative(s) |
| --- | --- | --- |
| 1. Should health monitoring be done pre-flight or in-flight? | In-flight monitoring | Pre-flight only |
| 2. Which microphones to use? | USB microphones | MEMS microphones (feasability study?) high-end industrial microphones |
| 3. Should testing be done with the drone on a stand? | Yes, use a drone stand | Free-flight testing only |
| 4. Where to position microphones? | Position 1- close to motor | Position 2- slightly farther away from motor |
| 5. Define Fault |  |  |

## Table 2: Pros and Cons of Each Decision

| Decision | Pros | Cons |
| --- | --- | --- |
| In-flight monitoring | – Detects faults arising mid-flight (e.g., chipped prop, degraded motor). – Critical for “drone-in-a-box” and remote ops without visual inspection. – Enables real-time fail-safety (e.g., 3-motor fallback). | – Adds computational/sensor load in-flight. – Motors/props rarely degrade in one flight. – Catastrophic faults usually cause immediate crashes. |
| USB microphones | – Plug-and-play with Jetson. – Low cost, no complex interfacing. – Good for proof-of-concept. | – Designed for human audio, weak in low frequencies. – Bulkier than MEMS; less UAV-friendly long-term. |
| Drone stand | – Safer for induced faults (chipped props, faulty motors). – Prevents crashes during testing. – Easier, more repeatable data collection. | – Doesn’t capture real-world flight noise. – Some features may differ from free-flight. |
| Mic placement | – Very strong signal-to-noise ratio for motor acoustic signatures (bearings, coils, commutation). – Less affected by airflow/turbulence from propeller compared to ridge placement. – Easier to standardize across motors (symmetrical placement possible). | – High risk of mechanical vibration coupling into the mic, potentially masking acoustic features. – May overemphasize motor sounds while underrepresenting propeller-related faults (e.g., small chips). |
| Define Faults |  |  |