Dr. Kulkarni Meeting Takeaways

- In-flight VS Pre-flight

  - No use implementing an in-flight monitoring framework for motors:

    - Motors will not degrade in the span of 1 flight

    - Any issues with the motors within the span of 1 flight are likely sudden impact type: like the propeller breaking in flight or smoke particles in the bearing etc. at which point likely the drone will instantly crash- we will not be able to do anything.

  - For the battery problem, in-flight monitoring is good.

- Thinking of other factors to consider possibly- in addition to audio from motors.

- Batteries:

  - Better to use new batteries

    - Issue with using multiple pre-used batteries is that we do not know any history. This can be managed but it is better to not go down that rabbit hole.

    - Using new batteries- may need us to do a lot of discharge cycles to see complete degradation [but we will start to see degradation within few cycles and it might be ok to not reach complete degradation]

- Mics

  - USB vs MEMS

    - USB will likely perform better

    - Upon review of spectrograms, looks like it should work

    - NASA’s mics are $600 each and we do not need that level

  - Position

    - Expect very different results with props on

    - Try placing the mic at multiple locations [not just close vs far, but within close, different locations and angles will work differently, especially with these omnidirectional mics]

    - Possibility of using all mics on one motor at a time [and testing motors one by one instead of all at once]

      - This takes away the whole thing about noise from other motors interfering]

      - This is even more possible with the tests being pre-flight and not post-flight.

    - Thought of testing motors without props on when checking for bearing faults [thought was that props sound may dominate bearing fault signatures]- Rejected- without the load [i.e., props on the motor] it may mean we are not able to see the fault at all.

- Next Steps:

  - Pre-flight testing confirmation

  - Mic positions decision making

  - Testing all motors at once or one by one decision making

  - Testing field design and build [Ally- would help with mech design?]

  - Data acquisition [code + implementation]

  - Start battery data collection

  - EDA on battery data format, how to obtain & track logs